Name : Navirath H N

USN : 1BM23CS206

Q6. minimize the makespan in a manufacturing environmet by optimzing the sequence of taks on multiple machines, considering each jobs specific task order and machine requiremnts using parallel cellular algorithm

CODE

import random

import numpy as np

# ---------- Problem Setup ----------

jobs = {

# job_id: [(machine_id, processing_time), ...]

0: [(0, 3), (1, 2), (2, 2)],

1: [(0, 2), (2, 1), (1, 4)],

2: [(1, 4), (2, 3), (0, 1)]

}

num_jobs = len(jobs)

num_machines = 3

# ---------- Makespan Calculation ----------

def calculate_makespan(sequence):

machine_time = [0] * num_machines

job_time = [0] * num_jobs

for job_id in sequence:

for machine_id, process_time in jobs[job_id]:

start = max(machine_time[machine_id], job_time[job_id])

finish = start + process_time

machine_time[machine_id] = finish

job_time[job_id] = finish

return max(machine_time)

# ---------- Initialize PCA Grid ----------

grid_size = 5

alpha = 0.5  # influence factor

iterations = 50

# Each cell holds a random job sequence

def random_sequence():

seq = list(jobs.keys())

random.shuffle(seq)

return seq

grid = [[random_sequence() for _ in range(grid_size)] for _ in range(grid_size)]

# ---------- Parallel Cellular Algorithm ----------

def get_neighbors(i, j):

neighbors = []

for di, dj in [(-1,0),(1,0),(0,-1),(0,1)]:

ni, nj = i + di, j + dj

if 0 <= ni < grid_size and 0 <= nj < grid_size:

neighbors.append((ni, nj))

return neighbors

for it in range(iterations):

new_grid = [[list(cell) for cell in row] for row in grid]

for i in range(grid_size):

for j in range(grid_size):

current = grid[i][j]

best_neighbor = current

best_makespan = calculate_makespan(current)

# Find best neighbor

for ni, nj in get_neighbors(i, j):

n_seq = grid[ni][nj]

n_makespan = calculate_makespan(n_seq)

if n_makespan < best_makespan:

best_makespan = n_makespan

best_neighbor = n_seq

# Update cell by mixing with best neighbor

if best_neighbor != current:

seq1, seq2 = current[:], best_neighbor[:]

cut = int(alpha * num_jobs)

new_seq = seq1[:cut] + [x for x in seq2 if x not in seq1[:cut]]

new_grid[i][j] = new_seq

grid = new_grid

# ---------- Find Best Schedule ----------

best_seq = None

best_makespan = float('inf')

for i in range(grid_size):

for j in range(grid_size):

m = calculate_makespan(grid[i][j])

if m < best_makespan:

best_makespan = m

best_seq = grid[i][j]

# ---------- Output ----------

print("Best job sequence:", best_seq)

print("Minimum makespan:", best_makespan)

OUTPUT :

Best job sequence: [1, 0, 2]

Minimum makespan: 11